Task 1:

Local Hosts File DNS Resolution Failure Simulation ([www.spotify.com](https://www.spotify.com))

Objective: Simulate a localized DNS resolution failure by editing the operating system's static hosts mapping file, overriding legitimate DNS lookup for [www.spotify.com](https://www.spotify.com) with an unroutable IP address, and documenting the resulting ping error response.

1. Step-by-Step Simulation Procedure:

1.Launch Notepad as Administrator:Step 1: Open Hosts File with Administrator Privileges.

- Search for Notepad in Windows Search → Right-click and select Run as administrator.

- Open File: C:\Windows\System32\drivers\etc\hosts (Select All Files .).

2.Inject Faulty IP Pointer:Step 2: Add Malformed Static Mapping.

- Append the following bogus entry at the bottom of the hosts file:

- Save the file (Ctrl + S).

3.Purge Local Resolver Cache:Step 3: Flush DNS Cache.

- Open Command Prompt (CMD) and execute:

4.Test Domain Reachability:Step 4: Execute Ping Command & Capture Error.

- Run the ping diagnostic command in CMD:

2. Documented Terminal Command & Error Output:

C:\Users\Admin> ipconfig /flushdns

Windows IP Configuration

Successfully flushed the DNS Resolver Cache.

C:\Users\Admin> ping www.spotify.com

Pinging www.spotify.com [192.0.2.1] with 32 bytes of data:

Request timed out.

Request timed out.

Request timed out.

Request timed out.

Ping statistics for 192.0.2.1:

Packets: Sent = 4, Received = 0, Lost = 4 (100% loss),

3. Root Cause Analysis & Findings:

- Observed Behavior: Instead of querying public DNS servers for Spotify's actual IP, the operating system checked the local hosts file first (as per standard OS name resolution order: Hosts File → DNS Cache → DNS Server).

- Error Analysis: The hosts entry forced the system to send ICMP Echo requests to 192.0.2.1. Since this IP address is unroutable, the ping operation failed with 100% packet loss (Request timed out).

Task 2 :

DHCP Incorrect Default Gateway Misconfiguration Simulation & Remediation

Objective: Intentionally misconfigure the Default Gateway parameter within a Cisco Packet Tracer DHCP pool, verify the bad gateway assignment on client PCs, demonstrate diagnostic identification steps (ipconfig /all and tracert), and document the step-by-step remediation procedure.

1. Misconfiguration Simulation Steps:

- Open DHCP Server in Cisco Packet Tracer → Services tab → DHCP.

- Configure the DHCP pool parameters intentionally with an incorrect Gateway:

  - Pool Name: serverPool

  - Default Gateway: 192.168.1.254 (INCORRECT - Actual Router Interface is 192.168.1.1)

  - DNS Server: 8.8.8.8

  - Start IP Address: 192.168.1.10

  - Subnet Mask: 255.255.255.0

- Click Save.

2. Client Verification & Problem Identification:

On PC0 (Client PC), open Command Prompt and execute network diagnostics:

C:\> ipconfig /all

FastEthernet0 Connection-specific DNS Suffix . :

Physical Address. . . . . . . . . . . . . . : 0060.2F29.412B

DHCP Enabled. . . . . . . . . . . . . . . . : Yes

IP Address. . . . . . . . . . . . . . . . . : 192.168.1.10

Subnet Mask . . . . . . . . . . . . . . . . : 255.255.255.0

Default Gateway . . . . . . . . . . . . . . : 192.168.1.254

DNS Server. . . . . . . . . . . . . . . . . : 8.8.8.8

C:\> ping 8.8.8.8

Pinging 8.8.8.8 with 32 bytes of data:

Request timed out.

Request timed out.

Destination host unreachable.

- Symptom: Local IP allocation (192.168.1.10) is successful, but all external network traffic and internet access fail because packets are sent to a non-existent gateway IP (192.168.1.254).

3. Step-by-Step Remediation Procedure:

1.Run Diagnostic Commands:Step 1: Identify Bad Gateway.

- Execute ipconfig /all on the client PC and compare the assigned Default Gateway against the actual router interface IP.

- Execute tracert 8.8.8.8 to confirm packet drop occurs right at the first hop.

2.Update DHCP Gateway Pool Parameter:Step 2: Correct Server Configuration.

- Open DHCP Server → Services → DHCP.

- Change Default Gateway from 192.168.1.254 to 192.168.1.1.

- Click Save.

3.Release & Renew DHCP Lease:Step 3: Refresh Client Network Configuration.

* Open Command Prompt on PC0 and execute:

<pre><code>ipconfig /release

ipconfig /renew

4.Confirm Reachability:Step 4: Verify Connectivity Restoration.

- Verify updated Gateway: Default Gateway: 192.168.1.1.

- Test ping connectivity: ping 8.8.8.8 → Reply from 8.8.8.8: bytes=32 time=1ms TTL=128 (SUCCESS).

4. Summary Results Table:

| Configuration State | Assigned Client IP | Assigned Gateway | Gateway Status | Ping External Target (8.8.8.8) |
| --- | --- | --- | --- | --- |
| Before Fix (Misconfigured) | 192.168.1.10 | 192.168.1.254 | Unreachable / Invalid | FAILED (Request timed out) |
| After Fix (Remediated) | 192.168.1.10 | 192.168.1.1 | Active Router Interface | SUCCESS (Reply received) |

Task 3 :

Diagnosing & Resolving Name Resolution Failure Using nslookup / dig

Objective: Troubleshoot a network issue where client hosts can access web destinations via IP address but fail using domain names, isolate the DNS failure using CLI diagnostic tools (nslookup and dig), and document the resolution steps.

1. Problem Symptom & Diagnostic Scenario:

- Observed Symptom:

  - Ping to direct IP address: ping 8.8.8.8 → SUCCESS (4 packets received)

  - Ping to domain name: ping [www.google.com](https://www.google.com) → FAILED ("Ping request could not find host www.google.com")

- Root Cause Identification: Network Layer 3 connectivity is functional, but the Application Layer DNS Name Resolution service is unreachable or misconfigured on the client host.

2. Diagnostic Execution (nslookup & dig Tools):

Step A: Running nslookup Diagnostics

C:\Users\Admin> nslookup www.google.com

DNS request timed out.

timeout was 2 seconds.

Server:  UnKnown

Address:  192.168.1.250  <-- (Misconfigured / Invalid Local DNS Server IP)

*** Request to UnKnown timed-out

Step B: Running dig Diagnostics (Linux/macOS)

dig @192.168.1.250 www.google.com

; <<>> DiG 9.18.18 <<>> @192.168.1.250 www.google.com

;; connection timed out; no servers could be reached

- Diagnostic Finding: The client machine is attempting to send DNS queries to an invalid or offline DNS server address (192.168.1.250).

3. Step-by-Step Troubleshooting Procedure & Solution:

1.Test Alternative DNS Server via nslookup:Step 1: Verify Direct Server Reachability.

- Query a known public DNS server (e.g., Google DNS 8.8.8.8) directly:

- Output: Returns correct IP (e.g., 142.250.190.46), proving local internet access is working and only client DNS configuration is faulty.

2.Re-assign Valid DNS Server Address:Step 2: Update Client TCP/IP DNS Configuration.

- Method A (Static IP Setup): Go to Network Adapter Properties → IPv4 → Change Preferred DNS Server to 8.8.8.8 (or local DNS server IP 192.168.1.1).

- Method B (DHCP Setup): Update the DNS Server parameter in the router/DHCP Server scope to distribute 8.8.8.8 automatically.

3.Purge Stale DNS Entries:Step 3: Flush DNS Resolver Cache.

- Open Command Prompt as Administrator and run:

* Execute `nslookup [www.google.com](https://www.google.com)` again:

<pre><code>Server:  dns.google

Address: 8.8.8.8

Name: www.google.com

Addresses: 142.250.190.46

* Test web access by domain name in browser or terminal → RESOLVED.

4. Summary Table:

| Diagnostic Stage | Command Executed | Observed Result | Resolution Status |
| --- | --- | --- | --- |
| Initial Test | nslookup [www.google.com](https://www.google.com) | DNS request timed out (IP 192.168.1.250) | Isolated Bad DNS IP |
| Target Test | nslookup [www.google.com](https://www.google.com) 8.8.8.8 | Non-authoritative answer: 142.250.190.46 | Public DNS Working |
| Post-Remediation | ipconfig /flushdns & nslookup | Addresses: 142.250.190.46 | FULLY RESOLVED |

Task 4 :

Troubleshooting APIPA (169.254.x.x) IP Fallback Issues & Resolution Guide

Objective: Document the underlying root causes when a client host fails to lease an IP address from a DHCP server and defaults to an APIPA address (169.254.0.0/16), and provide a structured, step-by-step troubleshooting workflow to restore dynamic network addressing.

1. Primary Root Causes for APIPA Fallback:

- Physical Layer / Connectivity Failure: Damaged Ethernet cables, disconnected switch ports, disabled Network Interface Cards (NIC), or incorrect Wi-Fi SSIDs/security keys preventing host-to-server communication.

- DHCP Server Service Unavailability: The DHCP daemon/service (DHCP Server in Windows or isc-dhcp-server/bind in Linux) is stopped, crashed, or the server machine is powered off.

- DHCP Scope Pool Exhaustion: All available host IP addresses within the active DHCP pool scope have been leased to existing clients, leaving no free IPs for new DHCPDISCOVER requests.

- Network Segmentation / VLAN / Firewall Block: The client is isolated in a separate VLAN without an operational DHCP Relay Agent (IP Helper Address) configured on the default gateway router, or firewall rules block UDP Ports 67/68.

2. Step-by-Step Resolution Workflow:

1.Verify Interface & Media Status:Step 1: Check Physical Link & Layer 2 Connectivity.

- Inspect network cable connection, link lights on switch ports, or active Wi-Fi connection.

- Ensure the client network adapter is enabled in Control Panel / Network Settings.

- Test link status via CLI: ipconfig /all (Ensure state is not Media disconnected).

2.Audit DHCP Daemon Health:Step 2: Verify DHCP Server Availability & Service Status.

- Log into the DHCP Server / Router interface.

- Verify the DHCP service is actively running:

  - Windows Server: Get-Service dhcpserver

  - Linux Server: sudo systemctl status isc-dhcp-server

- If stopped, restart the service: sudo systemctl restart isc-dhcp-server.

3.Check Scope Exhaustion:Step 3: Audit DHCP Scope & Pool Capacity.

- Open DHCP Management Console and check active address leases.

- If 100% of addresses are consumed:

  - Expand the IP pool subnet mask (e.g., extend range from 192.168.1.10–192.168.1.50 to 192.168.1.10–192.168.1.200).

  - Reduce lease time (e.g., from 8 days to 8 hours for dynamic/guest environments).

  - Reclaim/clear inactive stale leases from the active lease table.

4.Check Cross-VLAN Relay Settings:Step 4: Verify Router DHCP Relay / IP Helper Address.

- If client and DHCP server reside on different VLANs, verify the router interface includes an IP Helper Address entry pointing to the DHCP server IP:

* On the client machine, open Command Prompt as Administrator and run:

<pre><code>ipconfig /release

ipconfig /renew

* Verify the client exits APIPA mode and acquires a valid network IP (e.g., 192.168.1.15) → RESOLVED.

3. APIPA Troubleshooting Summary Table:

| Diagnostic Stage | Verification Check | Remediation Action | Expected Outcome |
| --- | --- | --- | --- |
| Stage 1: Client | ipconfig /all | Re-enable NIC / Reconnect Cable | Link Layer UP |
| Stage 2: Server | systemctl status isc-dhcp-server | Restart DHCP Service / Enable Pool | Server listening on Port 67 |
| Stage 3: Pool Scope | Active Lease Count | Expand IP Pool Range / Lower Lease Duration | Available IP Space > 0 |
| Stage 4: Client Refresh | ipconfig /renew | Force new DORA Broadcast process | IP changed from 169.254.x.x to 192.168.1.x |

Task 5:

Troubleshooting Documentation: Common DNS & DHCP Network Issues

Objective: Document a comprehensive troubleshooting matrix detailing 3 prevalent Domain Name System (DNS) faults and 3 Dynamic Host Configuration Protocol (DHCP) operational failures, outlining their observed symptoms, underlying root causes, and verified technical resolution procedures.

1. Common DNS Network Issues Matrix:

| Issue # | DNS Problem / Fault | Symptoms Observed | Possible Root Causes | Recommended Technical Solution |
| --- | --- | --- | --- | --- |
| DNS 1 | Name Resolution Failure | Clients can ping IP addresses (e.g., 8.8.8.8) but web browsers report "Server IP address could not be found" (ERR_NAME_NOT_RESOLVED). | • Incorrect DNS server IP configured on client.• DNS service daemon crashed.• Firewall blocking UDP Port 53. | • Verify client Preferred DNS settings (8.8.8.8).• Restart DNS service (sudo systemctl restart bind9).• Allow Port 53 through UFW/firewall. |
| DNS 2 | Stale / Poisoned Local DNS Cache | Browsers redirect to old IP address after domain IP change or show incorrect site version. | Outdated resource records cached locally in the OS or browser DNS resolver cache. | Purge local DNS cache on client terminal using ipconfig /flushdns (Windows) or sudo resolvectl flush-caches (Linux). |
| DNS 3 | Zone File Syntax Error | DNS server fails to load zone file (musicmania.in) or returns SERVFAIL status to clients. | Missing semicolons, trailing dots missing in FQDNs, or updated zone serial number not incremented. | Validate zone file syntax using named-checkzone musicmania.in /etc/bind/db.musicmania.in, fix typos, increment serial number, and reload BIND. |

2. Common DHCP Network Issues Matrix:

| Issue # | DHCP Problem / Fault | Symptoms Observed | Possible Root Causes | Recommended Technical Solution |
| --- | --- | --- | --- | --- |
| DHCP 1 | APIPA Fallback (169.254.x.x) | Client loses intranet/internet connectivity and auto-assigns an IP in the 169.254.0.0/16 range. | • DHCP server daemon stopped/unreachable.• Network cable disconnected.• DHCP scope pool exhausted. | Check physical connection, ensure DHCP service is running, expand active pool scope size, and execute ipconfig /renew. |
| DHCP 2 | Incorrect Default Gateway Assignment | Client acquires local IP (192.168.1.10) but cannot reach remote networks or external internet. | Default Gateway IP misconfigured in the DHCP pool options (e.g., set to an unassigned IP). | Edit DHCP server pool options, correct the Default Gateway parameter to match the active router LAN IP (192.168.1.1), and refresh client lease. |
| DHCP 3 | Duplicate IP Address Conflict | OS displays "Windows has detected an IP address conflict" message; intermittent packet loss occurs. | • Static IP assigned to a device within the dynamic DHCP scope range.• Stale lease table entries. | Exclude static device IPs from the DHCP pool range (ip dhcp excluded-address) or enable DHCP conflict detection on the server. |

3. Troubleshooting Summary Conclusion:

Maintaining network reliability requires systematic isolation of protocol-specific failures. Layer 3/4 network connectivity must be verified first using ping and tracert. For DNS, configuration syntax and Port 53 reachability are key; for DHCP, monitoring lease pool exhaustion, conflict exclusions, and correct option parameters (Gateway & DNS) ensures smooth client onboarding.