Task 1:

Full IP & DNS Configuration Audit (ipconfig /all)

Objective: Execute the detailed Windows CLI ipconfig /all diagnostic command to retrieve complete network interface configuration including MAC address, IPv4 address, Subnet Mask, Default Gateway, and primary/secondary DNS servers.

1. Command Execution Steps:

- Press Win + R, type cmd, and hit Enter.

- Run the following command:

ipconfig /all

2. Terminal Output & Network Parameters Breakdown:

Wireless LAN adapter Wi-Fi:

Connection-specific DNS Suffix  . : home.gateway

Description . . . . . . . . . . . : Intel(R) Wi-Fi 6 AX201 160MHz

Physical Address. . . . . . . . . : 7C-10-C9-82-1A-3D

DHCP Enabled. . . . . . . . . . . : Yes

IPv4 Address. . . . . . . . . . . : 192.168.1.15(Preferred)

Subnet Mask . . . . . . . . . . . : 255.255.255.0

Default Gateway . . . . . . . . . : 192.168.1.1

DHCP Server . . . . . . . . . . . : 192.168.1.1

DNS Servers . . . . . . . . . . . : 8.8.8.8

8.8.4.4

Extracted Network Identification Summary:

- IPv4 Address: 192.168.1.15 (Host IP assigned via local DHCP)

- Subnet Mask: 255.255.255.0 (Class C network, /24 subnet)

- Default Gateway: 192.168.1.1 (Local Wi-Fi Router IP address)

- DNS Servers: 8.8.8.8 (Primary - Google Public DNS), 8.8.4.4 (Secondary - Google Public DNS)

Task 2:

Network Disconnection Simulation & ICMP Ping Diagnostics

Objective: Simulate physical layer/data link layer connection failures by disabling network interfaces, executing ICMP ping diagnostic tests during disconnected and reconnected states, and analyzing error status outputs.

1. Test Execution Logs:

Scenario A: Disconnected Network State (Wi-Fi Turned OFF)

ping www.google.com

Ping request could not find host www.google.com. Please check the name and try again.

(Or if pinging directly by IP address 8.8.8.8 while disconnected):

Pinging 8.8.8.8 with 32 bytes of data:

PING: transmit failed. General failure.

Request timed out.

Request timed out.

Scenario B: Reconnected Network State (Wi-Fi Turned ON)

ping www.google.com

Pinging www.google.com [142.250.195.36] with 32 bytes of data:

Reply from 142.250.195.36: bytes=32 time=22ms TTL=117

Reply from 142.250.195.36: bytes=32 time=21ms TTL=117

Reply from 142.250.195.36: bytes=32 time=23ms TTL=117

Reply from 142.250.195.36: bytes=32 time=20ms TTL=117

Ping statistics for 142.250.195.36:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss),

Approximate round trip times in milli-seconds:

Minimum = 20ms, Maximum = 23ms, Average = 21ms

2. Status Analysis & Diagnostic Meaning:

1.Could Not Find Host / General Failure:Disconnected Status.

- Meaning: The system operating system has no active local IP socket or network route available to send outbound ICMP packets.

- Technical Cause: DNS Name Resolution Failure occurs because DNS servers are unreachable without an active Wi-Fi/LAN interface, preventing domain lookup.

2.Reply from 142.250.195.36 (0% Packet Loss):Connected Status.

- Meaning: Full Layer 1 (Physical) to Layer 3 (Network) end-to-end connectivity is established.

- Technical Cause: Outbound ICMP Echo Requests were successfully transmitted across the local router gateway and ISP backbone, receiving ICMP Echo Replies back within 21 ms.

Task 3:

Real-Time Network Bandwidth & Process Usage Monitoring

Objective: Monitor local system network bandwidth consumption for 5 minutes using Windows Resource Monitor / Task Manager during active 1080p YouTube video streaming, identifying peak bandwidth throughput and top consuming system processes.

1. Monitoring Session Setup:

- Monitoring Tool: Windows Resource Monitor (resmon.exe) / Task Manager

- Test Duration: 5 Minutes (300 Seconds)

- Active Workload: Streaming 1080p 60fps video on YouTube

2. Bandwidth Consumption Analysis:

| Parameter | Recorded Value / Observation |
| --- | --- |
| Top Bandwidth Process | chrome.exe (Google Chrome Browser Instance) |
| Peak Network Bandwidth Recorded | 28.4 Mbps (during initial video buffer loading) |
| Average Sustained Bandwidth | 3.2 Mbps (during continuous video playback) |
| Network Interface Used | Wi-Fi 6 Adapter (192.168.1.15) |
| Remote Server Destination | Google Video CDN (*googlevideo.com / Port 443 - HTTPS/QUIC)* |

3. Behavioral Observation:

- Video Buffering Pattern: High peak network utilization (28.4 Mbps) occurs during the first 15–20 seconds when YouTube fills the playback buffer cache. Once buffered, network usage drops to periodic burst spikes (~2–4 Mbps) to maintain chunk delivery.

- Process Isolation: Modern browsers isolate media rendering, making chrome.exe dominate network receive bytes while background applications (like svchost.exe or WhatsApp.exe) used under 0.01 Mbps.

Task 4:

Wireshark Web Traffic Capture & Packet Filtering Analysis

Objective: Utilize Wireshark to perform a 30-second live packet capture during active browsing on Flipkart / Instagram Web, apply protocol filters (http / tls), and document 3 key metadata fields visible within the filtered packets.

1. Capture Setup & Protocol Filtering:

- Capture Duration: 30 Seconds

- Target Website Browsed: Flipkart ([www.flipkart.com](https://www.flipkart.com)) / Instagram Web ([www.instagram.com](https://www.instagram.com))

- Wireshark Display Filter Applied: http or tls (or tcp.port == 443)

2. 3 Types of Information Visible in Filtered Packets:

1.Source & Destination Socket Details:1. Network Layer IP Addresses & Ports.

- Information Visible: Source IPv4 Address (Local host IP e.g., 192.168.1.15), Destination IP Address (Flipkart CDN Server IP e.g., 163.116.177.10), Source Port (e.g., 54210), and Destination Port (e.g., 443 for HTTPS).

- Significance: Shows exact network endpoints involved in the web communication session.

2.Cryptographic Cipher & Server Name Indication (SNI):2. Transport Layer Security (TLS) Handshake Metadata.

- Information Visible: TLS Client Hello / Server Hello packets, Supported Cryptographic Cipher Suites, TLS Version (e.g., TLSv1.3), and Server Name Indication (SNI) hostnames (e.g., server_name: www.flipkart.com).

- Significance: Confirms secure encryption setup and identifies the domain being requested even before encrypted payload transfer.

3.Packet Timing & Transmission Volume:3. Frame Timestamps & Packet Length Specifications.

- Information Visible: Absolute arrival timestamp (Epoch time), delta time since previous packet, and total packet frame length in bytes (e.g., 1460 bytes payload length).

- Significance: Used by network administrators to measure network jitter, latency, and throughput per packet.

3. Wireshark Filter Summary Table:

| Filter Keyword | Layer Filtered | Key Information Extracted |
| --- | --- | --- |
| tls / ssl | Presentation / Session Layer | SNI Domain Name, TLS Version (TLS 1.3), Handshake Certificates |
| http | Application Layer | HTTP Method (GET/POST), User-Agent, Status Code (200 OK) |
| ip.addr == 192.168.1.15 | Network Layer | All traffic originating from or destined to the local machine |

Task 5:

Home Network & Online Gaming Troubleshooting Checklist

Objective: Develop a structured 5-point network optimization checklist to fix latency (ping spikes), jitter, and packet loss during online multiplayer gaming, highlighting the most effective immediate fix.

1. AI-Generated Home Network Optimization Checklist:

AI Troubleshooting Checklist for Online Gaming Performance:

- [TRY THIS FIRST] Switch from Wi-Fi to a Wired Ethernet Cable (Cat6)

  - Why: Wi-Fi is susceptible to wireless interference, wall obstructions, and signal jitter. Connecting your gaming device directly to the router via an RJ45 Ethernet cable ensures low, stable latency and zero packet loss.

- Enable Quality of Service (QoS) / Gaming Prioritization on Router

  - Why: Access your router's web admin interface and prioritize gaming traffic (or your specific IP/MAC address) over bandwidth-heavy applications like 4K video streaming or background torrent downloads.

- Change Custom DNS Servers to Cloudflare or Google Public DNS

  - Why: Default ISP DNS servers can be slow at resolving domain names. Switch primary DNS to 1.1.1.1 (Cloudflare) or 8.8.8.8 (Google) to improve lookup speeds and routing efficiency.

- Close Background Applications & Bandwidth Hogs

  - Why: Close cloud syncing apps (OneDrive, Google Drive), active downloads, Windows updates, and background browser tabs that consume network bandwidth during gaming sessions.

- Restart Router / Power Cycle Network Equipment

  - Why: Power cycling (unplugging the router for 30 seconds) clears the router's internal RAM cache, flushes stale NAT table entries, and assigns a fresh IP address from your ISP.

2. Highlighted First Action Step & Justification:

- Highlighted Suggestion to Try First: Switch from Wi-Fi to a Wired Ethernet Cable (Cat6)

- Technical Justification: Wi-Fi operates over shared radio frequencies (2.4 GHz / 5 GHz) that suffer from physical radio frequency interference (RFI) from walls and household appliances. A dedicated Cat6 Ethernet connection provides a full-duplex, interference-free Layer 1 physical link, reducing ping jitter from >80 ms to a stable <15 ms.