Task 1 :

Static IP Addressing Assignment & Inter-Device Ping Connectivity Verification

Objective: Configure static IPv4 network parameters across five end-user host devices (PCs/Laptops) within the same local IP subnet (192.168.1.0/24) in a Small Office/Home Office (SOHO) topology, and verify mutual Layer 3 reachability using ICMP Echo Request (ping) tests.

1. Static IP Address Allocation Table:

| Device Hostname | Device Type | Connection Interface | Configured IP Address | Subnet Mask | Default Gateway | IP Allocation Mode |
| --- | --- | --- | --- | --- | --- | --- |
| PC-1 (Admin) | Desktop PC | FastEthernet0 | 192.168.1.11 | 255.255.255.0 | 192.168.1.1 | Static |
| PC-2 (Finance) | Laptop | FastEthernet0 | 192.168.1.12 | 255.255.255.0 | 192.168.1.1 | Static |
| PC-3 (HR) | Desktop PC | FastEthernet0 | 192.168.1.13 | 255.255.255.0 | 192.168.1.1 | Static |
| PC-4 (Sales) | Laptop | FastEthernet0 | 192.168.1.14 | 255.255.255.0 | 192.168.1.1 | Static |
| PC-5 (IT Support) | Desktop PC | FastEthernet0 | 192.168.1.15 | 255.255.255.0 | 192.168.1.1 | Static |

2. Configuration Commands & GUI Setup:

On PC-1 (Desktop Configuration):

- IP Address: 192.168.1.11

- Subnet Mask: 255.255.255.0

- Default Gateway: 192.168.1.1

On Linux CLI Alternative Setup (if applicable):

sudo ip addr add 192.168.1.11/24 dev eth0

sudo ip route add default via 192.168.1.1

3. ICMP Ping Connectivity Verification Tests:

Execution of ping sweeps from PC-1 (192.168.1.11) to all other 4 hosts:

C:\Users\Admin> ping 192.168.1.12

Pinging 192.168.1.12 with 32 bytes of data:

Reply from 192.168.1.12: bytes=32 time<1ms TTL=128

Reply from 192.168.1.12: bytes=32 time<1ms TTL=128

Ping statistics for 192.168.1.12:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss).

C:\Users\Admin> ping 192.168.1.13

Pinging 192.168.1.13 with 32 bytes of data:

Reply from 192.168.1.13: bytes=32 time<1ms TTL=128

Reply from 192.168.1.13: bytes=32 time<1ms TTL=128

Ping statistics for 192.168.1.13:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss).

C:\Users\Admin> ping 192.168.1.14

Pinging 192.168.1.14 with 32 bytes of data:

Reply from 192.168.1.14: bytes=32 time<1ms TTL=128

Reply from 192.168.1.14: bytes=32 time<1ms TTL=128

Ping statistics for 192.168.1.14:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss).

C:\Users\Admin> ping 192.168.1.15

Pinging 192.168.1.15 with 32 bytes of data:

Reply from 192.168.1.15: bytes=32 time<1ms TTL=128

Reply from 192.168.1.15: bytes=32 time<1ms TTL=128

Ping statistics for 192.168.1.15:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss).

4. Verification Conclusion:

Result: All 5 devices reside on the same broadcast domain (192.168.1.0/24). ICMP echo requests confirmed 0% packet loss across all devices, verifying healthy physical Layer 1/2 switch connections and valid Layer 3 static IP configuration.

Task 2:

DHCP Server Setup & Automated Dynamic IP Lease Verification

Objective: Deploy and configure a centralized DHCP Server service within the simulated Small Office LAN to dynamically distribute unique IPv4 addresses, subnet masks, default gateways, and DNS server parameters to three client host devices.

1. DHCP Server Pool Configuration:

- DHCP Service Status: Enabled (Active)

- Pool Name: SOHO_DHCP_Pool

- Default Gateway: 192.168.1.1

- DNS Server: 8.8.8.8

- Start IP Address: 192.168.1.100

- Subnet Mask: 255.255.255.0

- Maximum Number of Users: 50 (Pool Range: 192.168.1.100 – 192.168.1.149)

Cisco IOS Router Configuration Syntax (Alternative CLI Method):

Router(config)# ip dhcp pool SOHO_DHCP_Pool

Router(dhcp-config)# network 192.168.1.0 255.255.255.0

Router(dhcp-config)# default-router 192.168.1.1

Router(dhcp-config)# dns-server 8.8.8.8

Router(dhcp-config)# exit

Router(config)# ip dhcp excluded-address 192.168.1.1 192.168.1.15

2. Client Execution & Verification (ipconfig /renew):

Switch client network interface settings on PC-6, PC-7, and PC-8 from Static to DHCP.

Terminal Verification Logs:

C:\Users\Client1> ipconfig /all

FastEthernet0 Connection-specific DNS Suffix . :

DHCP Enabled. . . . . . . . . . . . . . . . : Yes

IPv4 Address. . . . . . . . . . . . . . . . : 192.168.1.100(Preferred)

Subnet Mask . . . . . . . . . . . . . . . . : 255.255.255.0

Default Gateway . . . . . . . . . . . . . . : 192.168.1.1

DNS Servers . . . . . . . . . . . . . . . . : 8.8.8.8

3. Dynamic Address Lease Allocation Table:

| Client Hostname | Interface MAC Address | Assigned Dynamic IPv4 Address | Subnet Mask | Default Gateway | DNS Server | Lease Allocation Status |
| --- | --- | --- | --- | --- | --- | --- |
| PC-DHCP1 | 000D.BD2A.1101 | 192.168.1.100 | 255.255.255.0 | 192.168.1.1 | 8.8.8.8 | Lease Active (DORA Success) |
| PC-DHCP2 | 000D.BD2A.1102 | 192.168.1.101 | 255.255.255.0 | 192.168.1.1 | 8.8.8.8 | Lease Active (DORA Success) |
| PC-DHCP3 | 000D.BD2A.1103 | 192.168.1.102 | 255.255.255.0 | 192.168.1.1 | 8.8.8.8 | Lease Active (DORA Success) |

4. Verification Conclusion:

Result: All three client devices successfully executed the DORA (Discover, Offer, Request, Acknowledge) protocol sequence. The DHCP server assigned unique, non-conflicting IP addresses within the designated 192.168.1.100/24 range along with correct subnet mask and gateway options.

Task 3:

Local DNS Service Configuration for Custom Hostname (musicplayer.local)

Local DNS Service Configuration for Custom Hostname (musicplayer.local)

Objective: Deploy a local DNS resolution service (using BIND9 / Router DNS / dnsmasq) to map the custom local hostname musicplayer.local to a local HTTP web server IP (192.168.1.50), enabling connected client devices to access the web interface via domain name instead of IP address.

1. Setup Architecture & Mapping Parameters:

- Target Hostname / FQDN: musicplayer.local

- Mapped Destination IPv4 Address: 192.168.1.50 (Local Web Server)

- DNS Server Address: 192.168.1.10 (or Router Gateway 192.168.1.1)

- Service Protocol: DNS (UDP/TCP Port 53) & HTTP (TCP Port 80)

2. Step-by-Step Configuration Guide:

Method A: BIND9 / Cisco Packet Tracer DNS Server Configuration

- Open the DNS Server (192.168.1.10) → Go to Services → DNS.

- Turn DNS Service ON.

- Create a new Resource Record:

  - Name: musicplayer.local

  - Type: A Record (Address Record)

  - Address: 192.168.1.50

- Click Add / Save.

Method B: dnsmasq Linux Router Setup (Alternative)

Edit /etc/dnsmasq.conf or /etc/hosts on the local router/gateway:

# Append local domain mapping

192.168.1.50   musicplayer.local

Restart dnsmasq service:

sudo systemctl restart dnsmasq

3. Verification Terminal & Browser Outputs:

Step 1: Domain Name Resolution Test via nslookup

On any connected LAN client PC (192.168.1.100), open Command Prompt:

C:\Users\Client1> nslookup musicplayer.local

Server:  soho-dns.local

Address:  192.168.1.10

Name:    musicplayer.local

Address:  192.168.1.50

Step 2: Web Browser Interface Test

Open Web Browser on client PC and navigate to [http://musicplayer.local](http://musicplayer.local):

HTTP Response Header: 200 OK

Content-Type: text/html

[ Web Browser Display Output ]

--------------------------------------------------

Welcome to Music Player Local Gateway Interface!

Server IP: 192.168.1.50

Status: Active & Stream Ready

4. Local Hostname Resolution Summary Table:

| Custom Hostname | Record Type | Mapped Target IP | Server Port | Verification Status |
| --- | --- | --- | --- | --- |
| musicplayer.local | A Record | 192.168.1.50 | 80 (HTTP) | SUCCESS (Resolved & Accessible) |

Task 4:

Layer 2 VLAN Segmentation Configuration & Isolation Testing

Objective: Implement Virtual Local Area Networks (VLAN 10 and VLAN 20) on a Cisco Managed Switch to segment network traffic into distinct broadcast domains, assign host interfaces, and demonstrate intra-VLAN reachability while proving inter-VLAN isolation.

1. VLAN Topology & Port Allocation Table:

| VLAN ID | VLAN Name | Assigned Switch Ports | Connected End Devices | Device IP Address | Subnet Mask |
| --- | --- | --- | --- | --- | --- |
| VLAN 10 | Workstations | FastEthernet 0/1 | PC-1 (Desktop) | 192.168.10.10 | 255.255.255.0 |
| VLAN 10 | Workstations | FastEthernet 0/2 | PC-2 (Laptop) | 192.168.10.11 | 255.255.255.0 |
| VLAN 20 | Smart_Devices | FastEthernet 0/3 | IoT-1 (Smart TV) | 192.168.20.10 | 255.255.255.0 |
| VLAN 20 | Smart_Devices | FastEthernet 0/4 | IoT-2 (Camera) | 192.168.20.11 | 255.255.255.0 |

2. Cisco Switch Command-Line Interface (CLI) Commands:

Switch> enable

Switch# configure terminal

! Step 1: Create VLAN 10 and assign Name

Switch(config)# vlan 10

Switch(config-vlan)# name Workstations

Switch(config-vlan)# exit

! Step 2: Create VLAN 20 and assign Name

Switch(config)# vlan 20

Switch(config-vlan)# name Smart_Devices

Switch(config-vlan)# exit

! Step 3: Assign Interfaces Fa0/1 and Fa0/2 to VLAN 10

Switch(config)# interface range fastEthernet 0/1 - 2

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 10

Switch(config-if-range)# exit

! Step 4: Assign Interfaces Fa0/3 and Fa0/4 to VLAN 20

Switch(config)# interface range fastEthernet 0/3 - 4

Switch(config-if-range)# switchport mode access

Switch(config-if-range)# switchport access vlan 20

Switch(config-if-range)# end

! Save Switch Configuration

Switch# copy running-config startup-config

3. Connectivity & Isolation Testing Results:

Test 1: Intra-VLAN Ping (PC-1 to PC-2 on VLAN 10)

C:\Users\PC-1> ping 192.168.10.11

Pinging 192.168.10.11 with 32 bytes of data:

Reply from 192.168.10.11: bytes=32 time<1ms TTL=128

Reply from 192.168.10.11: bytes=32 time<1ms TTL=128

Ping statistics for 192.168.10.11:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss).

- Result: SUCCESS. Devices residing within the same VLAN communicate natively across Layer 2 broadcast domains.

Test 2: Inter-VLAN Ping (PC-1 in VLAN 10 to IoT-1 in VLAN 20)

C:\Users\PC-1> ping 192.168.20.10

Pinging 192.168.20.10 with 32 bytes of data:

Request timed out.

Request timed out.

Request timed out.

Request timed out.

Ping statistics for 192.168.20.10:

Packets: Sent = 4, Received = 0, Lost = 4 (100% loss).

- Result: FAILED (100% Packet Loss). Traffic between VLAN 10 and VLAN 20 is completely blocked at Layer 2, demonstrating effective broadcast domain separation and enhanced network security.

4. Verification Summary Table:

| Source Device | Source VLAN | Target Device | Target VLAN | Ping Test Result | Network Status |
| --- | --- | --- | --- | --- | --- |
| PC-1 (192.168.10.10) | VLAN 10 | PC-2 (192.168.10.11) | VLAN 10 | Reply Received | Connected (Intra-VLAN) |
| IoT-1 (192.168.20.10) | VLAN 20 | IoT-2 (192.168.20.11) | VLAN 20 | Reply Received | Connected (Intra-VLAN) |
| PC-1 (192.168.10.10) | VLAN 10 | IoT-1 (192.168.20.10) | VLAN 20 | Request Timed Out | Isolated (Inter-VLAN Blocked) |

Task 5:

NAT (Network Address Translation) Overload / PAT Configuration & Internet Reachability Simulation

Objective: Configure Network Address Translation Overload (Port Address Translation - PAT) on the edge router to translate private LAN IP addresses (192.168.1.0/24) to a single public WAN IP address (203.0.113.2), enabling secure simulated internet reachability to external web servers (www.wikipedia.org).

1. Network Addressing Architecture:

- Internal LAN Network (Private IP): 192.168.1.0/24 (Interface Gig0/0 - Inside NAT)

- External WAN Network (Public IP): 203.0.113.0/30 (Interface Gig0/1 - Outside NAT)

- Simulated External Web Server: 203.0.113.100 (www.wikipedia.org)

- Default Route to ISP: ip route 0.0.0.0 0.0.0.0 203.0.113.1

2. Cisco Router CLI NAT Configuration Syntax:

Router> enable

Router# configure terminal

! Step 1: Configure Interface IP Addresses & Define NAT Roles

Router(config)# interface GigabitEthernet 0/0

Router(config-if)# ip address 192.168.1.1 255.255.255.0

Router(config-if)# ip nat inside

Router(config-if)# no shutdown

Router(config-if)# exit

Router(config)# interface GigabitEthernet 0/1

Router(config-if)# ip address 203.0.113.2 255.255.255.252

Router(config-if)# ip nat outside

Router(config-if)# no shutdown

Router(config-if)# exit

! Step 2: Configure Default Route toward ISP Gateway

Router(config)# ip route 0.0.0.0 0.0.0.0 203.0.113.1

! Step 3: Define Standard ACL permitting Private LAN Subnet

Router(config)# access-list 1 permit 192.168.1.0 0.0.0.255

! Step 4: Enable NAT Overload (PAT) on Outside WAN Interface

Router(config)# ip nat inside source list 1 interface GigabitEthernet 0/1 overload

Router(config)# end

Router# copy running-config startup-config

3. Verification & Diagnostic Test Logs:

Test A: Ping Test from LAN Device to External Server (203.0.113.100)

From internal client PC-1 (192.168.1.10):

C:\Users\PC1> ping www.wikipedia.org

Pinging www.wikipedia.org [203.0.113.100] with 32 bytes of data:

Reply from 203.0.113.100: bytes=32 time=12ms TTL=55

Reply from 203.0.113.100: bytes=32 time=11ms TTL=55

Ping statistics for 203.0.113.100:

Packets: Sent = 4, Received = 4, Lost = 0 (0% loss).

Test B: Verify Active NAT Translations on Router CLI

Execute on the Edge Router:

Router# show ip nat translations

Pro  Inside global     Inside local       Outside local      Outside global

icmp 203.0.113.2:1024  192.168.1.10:1024  203.0.113.100:1024 203.0.113.100:1024

tcp  203.0.113.2:80    192.168.1.10:49152 203.0.113.100:80   203.0.113.100:80

4. NAT Configuration Summary Table:

| Parameter Name | Assigned Interface / Value | Role in NAT Architecture |
| --- | --- | --- |
| NAT Inside Interface | GigabitEthernet 0/0 | Connects Internal LAN (192.168.1.0/24) |
| NAT Outside Interface | GigabitEthernet 0/1 | Connects External ISP / Public Internet |
| Permitted LAN ACL | access-list 1 permit 192.168.1.0/24 | Filters traffic eligible for NAT translation |
| Translation Type | NAT Overload (PAT) | Maps multiple private IPs to 1 public IP using port numbers |
| Verification Status | show ip nat translations | PASSED (Active Dynamic Translation) |